Dated: 14th June 2013

Sub: Technical evaluation of Bids for Enquiry No.6000/P063/012/PO63000021, dated 16/04/2013 for Procurement of 50 Nos. of PCs and Printers on buyback basis.

-------------------------------------------------------------------------------------------------------------------------

The clarifications/confirmations submitted by the vendors against our technical observations were  reviewed and our observations are as under:

The following URLs from Intel® were additionally referred in the context of the compatibility of H61 series motherboards with 3rd Generation i-5 Processors.

http://processormatch.intel.com/CompDB/SearchResult.aspx?ProcNbr=i5-3330

http://processormatch.intel.com/CompDB/SearchResult.aspx?ProcNbr=i5-3470

http://ark.intel.com/products/65509/Intel-Core-i5-3330-Processor-6M-Cache-up-to-3_20-GHz

http://ark.intel.com/products/68316/Intel-Core-i5-3470-Processor-6M-Cache-up-to-3_60-GHz

http://ark.intel.com/products/52806/Intel-BD82H61-PCH?q=H61

M/s. Jupiter and M/s. Computer shop have quoted ACER PCs having 3rd Generation i-5 Processors with Intel H61 series mother board.  As per Intel documentation on the websites, H-61 series motherboards are not categorized in the compatible matrix for the processors offered by the parties.  However, one internet link is showing they are partially compatible.  From this, it appears that the H-61 series motherboard is for 2nd generation processors and with addition of firmware patches some versions of H-61 series motherboards have been made compatible for 3rd generation processors.  These type of upgraded Mother Board/firmware and are not technically acceptable to us, as there are number of other Intel Chipsets series designed for 3rd generation processors (in which all versions compatible) are readily available.

In view of the above, both parties - M/s. Jupiter and M/s. Computer shop shall be asked to offer all versions fully compatible Intel® Chipset Motherboards which are compatible with 3rd Generation i-5 Processors offered by them.

- M/s. Jupiter Automation (Model: Acer Veriton M200 H61):

- The vendor has not submitted their clarifications/supporting documents from Intel® as requested in our earlier technical observations, sofar.  We have referred to various documents related to H-61 and Core i5 3470s processor and observed that H-61 series motherboards mainly compatible for 2nd Generation Intel® processors and some variants of H-61 series motherboards have been made compatible for 3rd generation processors with addition of firmware patches.  Such type of upgraded Mother Board/firmware and are not technically acceptable.   The vendor to be requested to submit their offer comprising Intel® make 3rd Generation Core i5 processor with fully compatible series Intel® make express chipset motherboards as per our NIT.

- The brochure page for Veriton M200-H61 model (page No.61) indicates that the DIMMs available for System Memory “2 DIMMs” whereas the vendor has confirmed in the Specification Confirmation Sheet – Clause No.7 for providing 4 DIMM slots as per NIT.  The  vendor to be requested to this point and reconfirm that their offered Veriton M200-H61 model motherboards have minimum 4 DIMM slots for System Memory as per NIT.

- M/s. Computer Shop (Model: Acer Veriton M200 H61):

- The vendor has not submitted their clarifications/supporting documents from Intel® as requested in our earlier technical observations, sofar.  We have referred to various documents related to H-61 and Core i5 3330 processor and observed that H-61 series motherboards mainly compatible for 2nd Generation Intel® processors and some variants of H-61 series motherboards have been made compatible for 3rd generation processors with addition of firmware patches.  Such type of upgraded Mother Board/firmware and are not technically acceptable.   The vendor to be requested to submit their offer comprising Intel® make 3rd Generation Core i5 processor with fully compatible series Intel® make express chipset motherboards as per our NIT.

- The brochure page for Veriton M200-H61 model (page No.72) indicates that the DIMMs available for System Memory “2 DIMMs” whereas the vendor has confirmed in the Specification Confirmation Sheet – Clause No.7 for providing 4 DIMM slots as per NIT.  The  vendor to be requested to this point and reconfirm that their offered Veriton M200-H61 model motherboards have minimum 4 DIMM slots for System Memory as per NIT.

- M/s. HCL  Info systems Ltd. (Model: HCL Infiniti M A390 Pro/Tru):

- The vendor has confirmed that they have now revised their offer for MicroSoft® Office Standard Edition 2010 32/64 bit (INDIC version) MOLP without Media 50 Nos. , as per NIT and the same is acceptable.

The clarification / confirmation as listed above from the concerned parties may be obtained for finalization for technical scrutiny.

(SUNILKUMAR VA)

Manager (MS)

CM (MS)

AGM (MS)