MS Department

KRISHAK BHARATI COOPERATIVE LIMITED

Scope of Work

Procurement of Laptop & A3 Laser MFP

Table of Contents

Part 1: SPECIFICATION CONFIRMATION SHEET	2

Part 2: SCOPE OF WORK	5

Scope of Work for Installation and Configuration of Laptop	5

Scope of Work for Installation and Configuration of A3 Colour Laser Multifunction Printer	5

1. Pre-Installation Activities	5

2. Installation	5

3. Configuration	5

3.4. User Access and Authentication	6

4. Testing and Verification	6

5. Training	6

6. Documentation and Handover	6

7. Post-Installation Comprehensive Warranty as per clause B-28 Part 1: Technical Confirmation Sheet	7

General Terms	7

Part 3: Schedule of Rates	8

# Part 1: SPECIFICATION CONFIRMATION SHEET

| SN | Specifications |  | Confirmation by Vendor (Yes / No) | Deviation, if any (Yes / No) |  | Remarks for the Deviation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Laptop – 1 (One) No. [Item Code: 1800003068] |  |  |  |  |  |  |  |
|  | Make/Model proposed | (HP/ DELL/ LENOVO) Business Laptop (Party to mention make and model) |  |  |  |  |  |  |
|  | Type | Thin & Light (Below 2 kg) |  |  |  |  |  |  |
|  | Processor | Intel® Core™ i5 – 12th Generation or above 64-bit Processor |  |  |  |  |  |  |
|  | Processor | Model Name |  |  |  |  |  |  |
|  | Chip set | Intel® make Chipset |  |  |  |  |  |  |
|  | TPM | Dedicated 2.0 or above |  |  |  |  |  |  |
|  | Graphics | Integrated Intel® HD Graphics or higher version |  |  |  |  |  |  |
|  | RAM | 16GB DDR4 or above |  |  |  |  |  |  |
|  | DIMM Slots | Minimum 2 |  |  |  |  |  |  |
|  | Hard Disk Drive | SSD Min 1 TB M.2 NVMe |  |  |  |  |  |  |
|  | Display | 14-inch FHD / UFHD |  |  |  |  |  |  |
|  | Resolution | 1920 x 1080 or above |  |  |  |  |  |  |
|  | Touch Screen | Not Required |  |  |  |  |  |  |
|  | Network RJ 45 | Integrated 10/100/1000 Mbps |  |  |  |  |  |  |
|  | Wireless LAN | Wi-Fi 6 and above |  |  |  |  |  |  |
|  | Bluetooth | V 5.0 or above |  |  |  |  |  |  |
|  | USB | Min. 1 USB Type-A port Min. 1 USB Type-C port |  |  |  |  |  |  |
|  | I/O Ports | HDMI; Head Phone |  |  |  |  |  |  |
|  | Web Camera | Minimum 720p Webcam |  |  |  |  |  |  |
|  | Microphone / Speakers | In Built |  |  |  |  |  |  |
|  | OS | Genuine MS® Win. 11 Pro. Edition 64 Bit (OEM Version) |  |  |  |  |  |  |
|  | Laptop Bag | Same as Laptop Brand |  |  |  |  |  |  |
|  | Warranty | One-year comprehensive on-site warranty from the date of successful installation |  |  |  |  |  |  |

| SN | Specifications |  | Confirmation by Vendor (Yes / No) | Deviation, if any (Yes / No) |  | Remarks for the Deviation |
| --- | --- | --- | --- | --- | --- | --- |
| B | A3 Color Laser MFP as per Specification – 1 (One) No. [Item Code: 1800045935] |  |  |  |  |  |
|  | Make/Model proposed | (HP/ CANON) |  |  |  |  |
|  | Functionality | Print, Copy, Scan |  |  |  |  |
|  | Print Technology | Laser |  |  |  |  |
|  | Type | Multifunctional (All-in-One) |  |  |  |  |
|  | Memory | 2 GB or Higher |  |  |  |  |
|  | Storage | 32 GB or Higher |  |  |  |  |
| Print Specifications |  |  |  |  |  |  |
|  | Print Speed | 26 PPM or higher (normal, A4) |  |  |  |  |
|  | Print Resolution(dpi) | 600 X 600 or higher |  |  |  |  |
|  | Duplex Printing | Automatic |  |  |  |  |
| Scan Specifications |  |  |  |  |  |  |
|  | Type | Flatbed; ADF (Duplex) |  |  |  |  |
|  | Scan Speed | 60 ipm or higher (normal, A4) |  |  |  |  |
|  | Scan Resolution(dpi) | 600 x 600 or higher |  |  |  |  |
|  | File Formats | JPEG, PDF, TIFF, PNG |  |  |  |  |
|  | Scan Feature | OCR (Optical Character Recognition) |  |  |  |  |
| Copy Specifications |  |  |  |  |  |  |
|  | Copy Speed | Same as print speed |  |  |  |  |
|  | Zoom Range | 25% - 400% |  |  |  |  |
|  | Resolution(dpi) | 600 X 600 or higher |  |  |  |  |
| Paper Handling |  |  |  |  |  |  |
|  | Paper Sizes | A3, A4, A5, A6, Letter, Legal |  |  |  |  |
|  | Paper Weight | 60 - 220 GSM |  |  |  |  |
|  | Input Capacity | O Standard Tray: 1000 sheets (2 Nos. 500 each) O Bypass Tray: 100 sheets O ADF: 50 sheets |  |  |  |  |
|  | Output Capacity | 250 sheets (A4) |  |  |  |  |
| Connectivity |  |  |  |  |  |  |
|  | Standard Interfaces | USB 2.0 Ethernet (10/100/1000Base-T) Wi-Fi |  |  |  |  |
|  | Driver Support | Windows, macOS, Linux |  |  |  |  |
|  | Mobile Printing | AirPrint, Google Cloud Print, Mopria |  |  |  |  |
| Display and Control |  |  |  |  |  |  |
|  | Screen | 7-inch color touchscreen LCD or better |  |  |  |  |
| General |  |  |  |  |  |  |
|  | Authentication and Access Control | PIN Login and Access Management System (Access Control) |  |  |  |  |
|  | Send Document | Email |  |  |  |  |
|  | Warranty | Three-year comprehensive on-site warranty from the date of successful installation |  |  |  |  |

# Part 2: SCOPE OF WORK

## Scope of Work for Installation and Configuration of Laptop

- Vendor shall carry out the installation of business laptop in premises of KRIBHCO.

- The pre-installed OS shall be updated to latest patch, if applicable.

- Update BIOS/firmware to the latest version, if required.

- Obtain acceptance certificate on the successful completion of the scope of work.

## Scope of Work for Installation and Configuration of A3 Color Laser Multifunction Printer

The scope of work outlines the necessary steps to install and configure an A3 Color laser multifunction printer (MFP) at a KRIBHCO site. The MFP includes printing, copying and scanning functionality.

### 1. Pre-Installation Activities

#### 1.1 Site Preparation

- Verify Site Readiness:

  - Confirm the availability of a stable and grounded power outlet within the required voltage range.

  - Ensure adequate ventilation and space for the printer to operate, including space for paper trays and access doors.

  - Check for a stable, level surface capable of supporting the printer's weight.

- Network and Connectivity:

  - Confirm the availability of an Ethernet port, Wi-Fi signal (if applicable), or USB connectivity as required.

  - Ensure necessary IP addresses and network details are available for configuration.

### 2. Installation

#### 2.1. Unpacking and Physical Setup

- Unpack and position the printer at the designated site.

- Insert toner cartridges, drum units, and any other consumables.

- Load paper into the appropriate trays and adjust the settings for the paper type and size.

2.2. Initial Hardware Configuration

- Power on the printer and run the initial setup wizard.

- Select language, date, time, and region settings.

- Update the firmware to the latest version if required.

### 3. Configuration

#### 3.1. Network Configuration

- Connect the printer to the network via Ethernet or Wi-Fi.

- Assign a static IP address provided by KRIBHCO Network team

- Test network connectivity.

#### 3.2. Driver and Software Installation

- Install printer drivers on client PCs or servers for all supported operating systems.

- Configure printer settings (default paper size, color, duplex printing, etc.) on the driver.

- Install additional software for advanced functionality, such as scanning application.

#### 3.3. Scanning Configuration

- Set up scanning to email:

  - Configure SMTP settings, including authentication if required.

- Set up scanning to network folders:

  - Create shared folders on the network and provide required permissions.

  - Configure SMB or FTP settings on the printer for folder access.

- Install and configure scanning software if applicable.

### 3.4. User Access and Authentication

- Configure user authentication (if required):

  - Set up PIN codes, LDAP integration, or user accounts.

### 4. Testing and Verification

- Print test pages to verify print quality and alignment.

- Test copying and scanning functionality.

- Ensure all PCs and devices on the network can connect to and use the printer.

- Verify advanced features such as duplex printing.

### 5. Training

- Provide basic user training, including:

  - Loading paper and replacing toner.

  - Operating the control panel for copying and scanning.

  - Troubleshooting basic issues (e.g., clearing paper jams).

- Provide administrator training for managing advanced settings.

### 6. Documentation and Handover

- Provide the following:

  - User manuals and guides.

- Obtain acceptance certificate on the successful completion of the scope of work.

### 7. Post-Installation Comprehensive Warranty as per clause B-28 Part 1: Technical Confirmation Sheet

- The Warranty clause No. B-28 for A3 Color Multifunction Printer shall include all parts and spares except toner cartridges in scope of vendor.

- Provide contact details for ongoing support or additional queries.

## General Terms

- Supply and installation of all the items at KRIBHCO’s Plant Site at Hazira.

- The vendor shall supply necessary cables for each item.

- Lodging and boarding expenses during entire period of installation and during warranty services shall be in the scope of the Party.

- Replacement of components if any shall be done with same or equivalent brand with the consent of KRIBHCO.

- The specifications for all the items shall be confirmed in our Specification Confirmation Sheet only.

- The vendor shall furnish OEM Authorization Letter towards this enquiry for supply and support of A3 Laser color MFP.

- Delivery Period shall be preferably within 4 weeks from receipt of P.O. and Installation of the same shall be carried-out within 4 weeks from receipt of material at KRIBHCO.

# Part 3: Schedule of Rates

| SN. | Description | Unit | Quantity | Rate ₹ | Amount ₹ |
| --- | --- | --- | --- | --- | --- |
| 1 | LAPTOP/ NOTEBOOK PC AS PER ATTACHED SPECIFICATION Supply, Installation, Configuration and Support for Business Laptop HP/DELL/Lenovo - Corei5 - 12th Generation - RAM - 16GB; SSD 1TB NVMe; 14" FHD; OS - Win.11Pro [KRIBHCO Item Code: 1800003068] | No. | 1 |  |  |
| 2 | A3 Color Laser Multifunctional Printer as per Specification Supply, Installation, Configuration and Support for A3 Color Laser Multifunctional Printer - Print/Copy/Scan, 26 PPM or higher, ADF [KRIBHCO Item Code: 1800045935] | No. | 1 |  |  |